Question Bank for

POWER ELECTRONICS

Q.1  Describe the switching characteristic of Power MOSFET and give applications with control requirements.

B) Give a comparison between MOSFET and IGBT.

Q.2 A) Describe the basic structure of MCT. Give its equivalent circuit and explain the turn on and turn off process.

B) Explain output and transfer characteristics of IGBT.

Q3 A) Explain four quadrant chopper feeding RLE load in detail.

B) What is time ratio control in dc choppers ? Explain the use of TRC for controlling the

output voltage in choppers.

Q.4  A) Explain the principle of operation of step up chopper. Derive expression for output voltage.

B) Draw a power circuit diagram for a type-A chopper. Show load voltage waveforms for

α = 0.3 and α= 0.8. For both these duty cycles, calculate : the average and rms values

of output voltage in terms of source voltage.

Q.5 A) What is a need for controlling the voltage at the output terminals of an inverter ? Describe

and compare various methods employed for the control of output voltage of inverter.

B) What is pulse width modulation ? Explain any one PWM technique in detail.

Q.6 A) Explain with circuit diagram and waveforms operation of single phase current source

inverter.

B) Explain working of three phase six step voltage source inverter in 180° mode of operation.

For star connected load draw output voltage waveforms. Show devices conducting in

each step.

PROF. HADPE S.S.